BSPD (Brake System Plausibility Device)

- Checks whether the driver is braking hard while the electrical system is still delivering substantial power to the motor

- If the driver is braking, the BSPD detects that and reduces power given to the motor

Requirements

- The vehicle must have a standalone nonprogrammable circuit to check for simultaneous braking and high power output

- The BSPD must Open the Shutdown Circuit EV.8.2.2 when the two of these exist for more than 0.5 sec:

  - Demand for hard braking

  - Tractive System current is at a level where 5 kW of electrical power in the DC circuit is delivered to the Motor(s) at the nominal battery voltage

- The BSPD must Open the Shutdown Circuit when there is an open or short circuit in any sensor input

How a circuit would recognize hard braking

- A brake-pressure sensor converts pressure into an electrical signal

- We would compare this signal with a voltage representing the hard-braking threshold using a comparator

  - There will be two signal inputs

    - + (non inverting): Sensor signal

    - - (inverting): Threshold with reference voltage

  - If the voltage of the sensor is higher than the threshold, the output is high (the sensor voltage is above the threshold and shutdown circuit must be used)

Comparators

- Compares 2 voltages and produces an output of High or Low to indicate which is larger

  - V+: Non-inverting input (+)

  - V-: Inverting input (-)

- If the non-inverting input is larger than the inverting input, the output will be high (opposite output would be low)

Op-amp (operational amp)

- Amplifies the output of the difference between the two input voltages